1. NOT Gate using NAND

A NAND gate with both inputs connected to the same input acts as a NOT gate.

Logic diagram

Both inputs of the NAND gate are connected to A.

Boolean expression

OUT=(A⋅A)′=A′

Truth Table

| A | OUT |
| --- | --- |
| 0 | 1 |
| 1 | 0 |

2. AND Gate using NAND

An AND gate can be constructed using two NAND gates.

Logic diagram

Boolean expression

First NAND:   X=(A⋅B)′

Second NAND:  OUT=(X⋅X)′=A⋅B

| A |  |  |  |  |  |  |  |  |  |  |  | B |  |  |  |  |  |  |  |  |  |  |  |  | OUT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 |  |  |  |  |  |  |  |  |  |  |  | 0 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
| 0 |  |  |  |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
| 1 |  |  |  |  |  |  |  |  |  |  |  | 0 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
| 1 |  |  |  |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  |  |  |  |  |  |  |  | 1 |

3. OR Gate using NAND

An OR gate can be constructed using three NAND gates.

Logic diagram

Boolean expression

First NAND: X=A′

Second NAND: Y=B′

Third NAND: OUT=(A′⋅B′)′

Using De Morgan's theorem: OUT=A+B​

| A |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | B |  |  |  |  |  |  |  |  |  |  |  |  | OUT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
| 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  |  |  |  |  |  |  |  | 1 |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |  |  |  |  |  |  |  |  |  |  |  |  | 1 |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  |  |  |  |  |  |  |  | 1 |